1 x DM32 WiFi Digital Gauge Package

  - DM32 WiFi Dual Channel Color Touchscreen Manometer

  - USB Cable, 6' (1.8 m)

  - Cat5 Cable, 7' (2 m) Blue

  - DM32 USB Charging Module (120V/60Hz, 240V/50Hz, or 110V/50Hz)

  - Gauge Case

  - DM32 Magnetic Clip

  - ISO17025 calibration of DM32

  - Gauge Calibration Verification Report

1 x 6000 Series Fan Set

      - Each fan set contains:

      - 6000 Series Calibrated Fan

      - High Power Drive for 6000 Series Fan (120V/60Hz, 240V/50Hz, or 110V/50Hz)

      - Range Ring A for 5000/6000 Series Fan

      - B8 Plate for 5000/6000 Series Fan

      - 8 x 4" Blank Plugs

      - 2 x Fan Cover

      - System Calibration Verification Report

1 x Regular Cloth Door Panel & Frame (new) - Single Fan

      - Aluminum Frame, Red Anodized

      - Cloth for 1000/2000/3000/5000/6000 Series Fan, Regular Height

      - Fan Crossbar Strap for Aluminum Frame

      - Case for Aluminum Frame

      - **Optional: 50" Large Aluminum Frame Offered At Additional Cost**

1 x Case for System Accessories

1 x Fan Case

1 x Blower Door Tubing Kit

1 x Umbilical for 1000/5000/6000 Series Fan, 7 ft (2 m)

| Model 6000 Fan |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Flow Into: | CFM | Litres/sec | M3/h |
| Max Flow, per fan (60 Hz) Reduces values by 15% for 50Hz fan | Free Air | 8,100 | 3,823 | 13,762 |
|  | 50 Pa | 7,700 | 3,634 | 13,082 |
|  | 75 Pa | 7,400 | 3,492 | 12,573 |
| Minimum Flow: |  |  |  |  |
| on Included Range B1 | 50 Pa | 95 | 49 | 161 |
| on Optional Range 74 | 50 Pa | 37 | 17 | 63 |
| on Optional Range 47 | 50 Pa | 21 | 10 | 35 |
| on Optional Range 29 | 50 Pa | 9 | 4 | 15 |
| Dimensions: | 22" (55.9 cm) inlet diameter, 9.75" (24.8 cm) long, 25" (63.5 cm) height |  |  |  |
| Weight: | 33.5 lb (15.2 kg) or 35.2 lb (16.0 kg) with Ring A & Plated B8 installed |  |  |  |
| Flow Accuracy: | +/-5% |  |  |  |
| Power: | 120V@60hz/22.0A or 240V@50hz/10.9A or 110V@50Hz/22.0A |  |  |  |
| Recommended Fan Calibration Interval: | 5 years |  |  |  |

| Digital Gauge Model: | DM32 WiFi |
| --- | --- |
| Link to Specifications: | Click here to see the full specs for the DM32 |

| Door Panel and Frame Model: | AL250 |
| --- | --- |
| Type of Panel: | Regular Cloth Panel & Frame – Single 1000/3000/5000/6000 Fan |
| Type of Frame: | Regular Sized Red Anodized Frame |
| Collapsed: | 27 1/8” X 53 3/8” (68.9cm X 135.6cm) |
| Expanded: | 43 5/8” X 97 ½” (110.8cm X 247.6cm) |
| Frame Thickness: | 1.75” (5.3 cm) |
| Weight: | 16 lbs (7.3 kg) |